Overhead Crane, Gantry and Rigging Policy

OVERVIEW

This policy applies to all contractors and employees at all locations that utilize cranes and rigging equipment for the movement or adjusting of objects by hoisting. Employees and contractors will not work around or with cranes unless they are properly trained for their level of interaction.

ACTIONS TO STAY SAFE

- Person in charge of lifting operations will have the knowledge to advise crews on load limits of lifting devices.

- Conduct and document inspections prior to use, including monthly/annually (rigging materials including ropes, chains, slings, etc. and crane components including outriggers, cables, blocks, hooks, etc.)

- Conduct pre-task risk assessments and implement critical controls.

- Ensure anti two-block devices are installed and functioning.

- Always use softeners to protect slings from damage.

- Crane operators will not engage in distracting activities.

- Anyone can give the signal to stop operations.

- No crane will be loaded beyond its capacity or used for other than its designated purpose.

- Ensure all moving parts are guarded if they expose employees to hazards.

- Complete critical lift plans where necessary (see Technical Supplement).

- All crane activity will utilize a designated signalperson or radios on a designated channel.

SUSPENDED LOADS

- No one is permitted to ride the hook, ball or any portion of a load.

- No one is allowed under a load unless it is effectively blocked from inadvertent movement.

- Use spotters, flagging or barricading to communicate the fall zone.

- Use push/pull sticks and tag lines whenever possible.

- Guiding a load into place by hand is only permitted when employees:

  - Have view of the height of the load

  - Understand potential pinch points and trip hazards

  - Understand the potential and actual swing hazards

  - Not at risk of being struck should the load fall

  - Maintain distance from the sling and load and between the sling and hook

OPERATOR EXPECTATIONS

- The operator has final responsibility and control over the crane operations.

- When two or more operators are required, one will be designated as the lead.

- Do not respond to unclear signals, or signals from anyone other than the designated signalperson (with the exception of STOP).

- Never intentionally ignore signals.

- All loads will be attached to the hook with a sling or other approved device.

- Position the hook over the load to prevent load swing.

- Properly seat rope in the drum and sheaves, ensuring line is not kinked or twisted (multiple part lines).

- Do not suddenly accelerate or decelerate a load, allow load to contact obstructions, swing over personnel, or allowing side loading or load dragging.

- When side pulling must occur, follow manufacturer recommendations and a written plan approved by a competent person.

OVERHEAD AND GANTRY CRANES

- Control boxes will be labeled for button action and direction.

- If there are two or more control boxes, all but one must be removed from service and locked, or procedures in place for pitch and catch activities.

INSPECTIONS

- Pre-use inspections

- Monthly inspections documented and retained for at least three (3) months.

- Quarterly inspection for severe use per ASME B30.2

- Annual inspection documented and retained for at least twelve (12) months.

- Reference 1910.179 for details for overhead cranes

CRANE CERTIFICATION, REPAIR

- Crane certifications must be current (within 12 months) and on-site, and a certification tag/label legible on equipment

- Recertify any crane that has been damaged or structurally repaired in any way.

- Inspect all crane equipment prior to use.

- Remove any damaged/defective crane equipment immediately and tag as out of service. Report to supervisors immediately.

- All repairs must be to manufacturer specifications, by qualified individuals, and include load testing if required.

RIGGING

- All rigging equipment will meet ANSI, ASME, or local equivalent requirements and not exceed designated working load limits (WLL).

- All rigging equipment will be rated. Capacity, material and manufacturer will be indicated on the equipment.

- Rigging assemblies will never be used in excess of the rated capacity of the weakest component.

- Rigging equipment used for towing will be clearly marked for that purpose and not used as rigging.

  - It will be inspected prior use per manufacturer recommendations and not used beyond capacity.

  - Conduct and document inspections prior to use, including monthly/annually

- Rigging equipment used as part of a fall protection system will be clearly marked for that purpose and not used as rigging.

  - It will be inspected prior use per manufacturer recommendations and not used beyond capacity.

  - Conduct and document inspections prior to use, including monthly.

INSPECTION, STORAGE AND REPAIR

- A qualified person will inspect all equipment and hoisting materials prior to use.

- Remove any damaged/defective lifting equipment immediately and tag as out of service or destroy. The manufacturer must complete all repairs.

- Maintain documented inspection logs. Inspect all hoisting equipment annually, and equipment used to hoist personnel every 6 months.

- Store in clean, orderly location, per manufacturer recommendations, and away from incompatible material and conditions.

- Do not store equipment on the floor, with liquids, or in direct sunlight.

LOAD RATING DETERMINATION

- Conducted by a qualified rigger prior to a lift.

- Will account for:

  - Weight, shape, and center of gravity of load to be lifted

  - Sling configuration while rigged

  - Stability of the load

  - Weight of all equipment including hoist blocks, headache balls, hooks and rigging and any other items added.

LOCALLY INSTALLED LIFTING POINTS

- All lifting points must meet ASME or equivalent standards.

- Use positive connection points (eyes, D-rings, shackles, etc.) in place of lifting lugs whenever possible.

- Only qualified welders can attach lifting eyes.

- A qualified person will determine eye location.

- When lifting points are welded, contact item manufacturer to determine the type of material you will be attaching lifting points.

- Load rating must be clearly marked on each lifting point.

- Manufacturer installed enclosed lifting points must be labeled as a lift point.